POWER

9-30V DC IN

Wi-Fi

NA

CELLULAR

5G/4G/3G/2G with Dual SIM, External 5dBi antenna (Optional 5G)

PROCESSOR

MediaTek MT7621 512MB DDR RAM, 64MB Flash

IDF05

# DATA SHEET

PRODUCT INFO

INTERFACE

3x LAN 10/100/1000, 1x WAN 10/100/1000, 2x RS485,4xDI/O, 4xAI, 1xUSB-H

OPERATING SYSTEM

Open-WRT Linux

Hardware Speciﬁcation

| CPU | MediaTek MT7621 |
| --- | --- |
| RAM | 512MB |
| FLASH | 64MB |
| WAN | 1x 10/100/1000 (Configurable WAN/LAN) |
| LAN | 3x 10/100/1000 |
| Serial Interface | 2x RS485,4xDI/O, 4xAI, 1xUSB-H |
| SIM | 2x Nano SIM (1x Active & 1x Passive) |
| Wi-Fi | 802.11 b/g/n/ac |
| RTC | Built-in RTC |
| Reset | Pinhole Reset Button |
| Antenna | Cellular: 2x SMA Connector, Wi-Fi: 4x SMA connector |
| Status LED | Signal strength, Wi-Fi 2.4 & Wi-Fi 5GHz LED, RS485 Rx & Tx LED, RS232 Rx & Tx LED, Power LED,1x ACT, 3x Prgm, 4x DI/O LED |
| Power Connector | Molex 4pin Sung Fit connector |
| Input Voltage Range | 9-30 Vdc |
| Power Adapter | N/A |
| Enclosure | Industrial Aluminium Enclosure |
| Mounting | DIN rail mounting Kit (Optional) |
| Design | Fanless |
| Dimensions | 110 x 44 x 94 mm (W x H x D) |
| Weight | 500 gm |
| Other | Grounding Screw |

| Storage Temperature | -30 ~ 70 ˚C |
| --- | --- |
| Operating Temperature | -20 ~ 60 ˚C |
| Humidity | 10 ~ 90% (non-condensing) |

Operational & Environmental

Cellular

SOFTWARE SPECIFICATIONS

| Cellular Module | 4G (LTE) - Cat 4 up to 150 Mbps, 3G - up to 42 Mbps, 2G - up to 236.8 Kbps (Optional 5G) |
| --- | --- |
| Status | Single strength (RSSI), SINR, RSRP, RSRQ |
| Band Management | Auto/User configurable |
| Operator Lock | Automatic/Manual |
| APN | Configurable, Auto APN |
| Modem conﬁguration mode | Single SIM, Dual SIM |
| SIM switching | Signal strength, ICMP echo, Data limit based |
| PDP Type | IPv4, IPv6, IPv4v6 |
| SIM Authentication | PAP,CHAP, No Authentication |
| SMS | Send SMS, Read SMS, SMS Commands |

Firewall & Networking

| Routing | Static routing, Advanced Static Routing, Dynamic Routing |
| --- | --- |
| Network Protocols | TCP, UDP, IPv4, IPv6, ICMP, NTP, HTTP, HTTPS, SCP, PPP, SSH, DHCP |
| Connection Monitoring | Ping Reboot, Scheduled Periodic Reboot |
| Firewall features | Port forward, trafﬁc rules, SNAT, Zone forwarding, Parental Control, Masquerading, MSS Clamping, Loopback IP, Loopback Rules. |
| Attack Prevention | SYN Flood Protection, Drop Invalid Packets |
| DHCP | Static and Dynamic |
| Multi-WAN Conﬁguration | Conﬁgurable Failover, load balancing between Cellular WAN, Wi-Fi WAN and Wired. |
| VLAN | Tagged VLAN, Untagged VLAN |
| VPN | Open VPN, IPsec, GRE, L2TP, PPTP, Wireguard, Zero Tier, IPIP Tunnel |
| Open VPN | TUN/TAP with TLS Certiﬁcates |
| IPsec | IKEv1, IKEv2, Aggressive/Main Mode |
| IPsec Encryption | 3DES, AES128, AES192, AES256 |
| IPsec Authentication | SHA1, MD5, SHA256, SHA384, SHA512 |
| IPsec DH Group | MODP768, MODP1024, MODP1536, MODP2048, MODP3072, MODP4096 |
| PPTP/L2TP | Server & Client with/without MPPE Encryption |
| Monitoring Application | Modem Monitoring with ICMP Echo, Router Monitoring with ICMP Echo |
| Time Synchronization | NTP Client, RTC, Manual |
| Diagnostic | Ping, Traceroute, Modem Debug Info, Iperf, Download Logs |
| Other Features | MAC binding, Connection Diagnostics, URL Filtering, Router Health Monitoring, Package Manager |

Remote Management

| NMS | Remote Monitoring & Management Platform for SILBO Networking products |
| --- | --- |
| NMS VPN | Remote Access of SILBO and non-SILBO Networking Devices |
| TR069 | TR069 Client for CPE Management |
| SMS Commands | Reboot Command, Uptime Command |

Gateway

| Gateway Protocols | Modbus TCP Slave, Modbus TCP Master |
| --- | --- |
| Serial Mode | Transparent (Over IP), Modbus Gateway |
| Data sender | HTTP, MQTT, Azure MQTT |
| Internal Storage | 4GB eMMC(optional) |

Operating System

| Operating System | Open-WRT based Linux OS |
| --- | --- |
| Supported Languages | Shell scripting, C, C++ |
| Development tools | SDK package with built environment provided (Optional) |

Standard Packaging

IDF05 Cellular Gateway with Industrial Aluminium Housing, 4x Wi-Fi Rubber Duck Antennas, 2x Cellular Rubber Duck Antennas, 0.5-meter Power Cord, 01-meter Ethernet cable, 2x RS485 TB,1xDIO TB, 1xAI TB,1x 24V ISO Output TB.

SPATIAL MEASUREMENT:

Top View

94 mm

110 mm

Front View

110 mm

44 mm

Rear View

110 mm

44 mm

v

*All rights reserved, subject to change without prior notice.

Get in touch:

sales@invendis.com

www.silbonetworks.com

Invendis Technologies India Private Limited

No. 230, 1st Cross, 38th Main, BOOHBCS Layout, BTM 2nd Stage, Bangalore – 560 068