POWER

Active PoE IN (IEEE 802.3af/at (48V-57V))

INTERFACE

1x 10/100/1000 (External) with PoE IN, 1 x 10/100/1000 internal for configuration

Wi-Fi

NA

PROCESSOR

MediaTek MT7621, 512MB DDR RAM, 32MB RAM

ODU-MT7621-01

# DATA SHEET

PRODUCT INFO

CELLULAR

4G/3G/2G with Dual SIM Active/Passive, Two Antennas , One IP66 External 5dBi main antenna and one 5dBI auxiliary internal Flexi Antenna

OPERATING SYSTEM

Open-WRT Linux

Hardware Speciﬁcation

| CPU | MediaTek MT7621 |
| --- | --- |
| RAM | 512MB |
| FLASH | 32MB |
| Ethernet | 1x 10/100/1000 (External) with PoE IN, 1 x 10/100/1000 internal for configuration |
| Cellular | 1x Onboard Quectel EC200A LTE 4G module |
| SIM | 2x Nano SIM with Tray (1x Active & 1x Passive) |
| Wi-Fi | NA |
| Reset | Pinhole Reset Button |
| Antenna | Cellular: 1x N Type Connector, 1x UFL connector |
| Status LED | 1x Power, 2 X Ethernet, 1 X LTE Activity, 1 X RGB LED to indicate Signal Strength, 1 X Dual Color SIM Presence |
| Power Connector | PoE in |
| Input Voltage Range | IEEE 802.3 af/at(48V-57V) |
| Power Adapter | No |
| Enclosure | ABS/PC IP66 Housing |
| Mounting | WALL Mount, Poll Mount |
| Design | Fanless |
| Dimensions | 140 x 75 x 224 mm (W x H x D) |
| Weight | 950 gm |

Operational & Environmental

| Storage Temperature | -20 ~ 60 ˚C |
| --- | --- |
| Operating Temperature | -0 ~ 50 ˚C |
| Humidity | 10 ~ 90% (non-condensing) |

Cellular

SOFTWARE SPECIFICATIONS

| Cellular Module | 4G (LTE) - Cat 4 up to 150 Mbps, 3G - up to 42 Mbps, 2G - up to 236.8 Kbps |
| --- | --- |
| Status | Single strength (RSSI), SINR, RSRP, RSRQ |
| Band Management | Auto/User configurable |
| Operator Lock | Automatic/Mannual |
| APN | Configurable, Auto APN, Dual APN (Under Development) |
| Modem conﬁguration mode | Single SIM, Dual SIM |
| SIM switching | Signal strength, ICMP echo, Data limit based |
| PDP Type | IPv4, IPv6, IPv4v6 |
| SIM Authentication | PAP,CHAP, No Authentication |
| Passthrough | IP Passthrough (Under Development) |
| Firewall & Networking SMS | Send SMS, Read SMS, SMS Commands |

| Routing | Static routing, Advanced Static Routing |
| --- | --- |
| Network Protocols | TCP, UDP, IPv4, IPv6, ICMP, NTP, HTTP, HTTPS, SCP, PPP, SSH, DHCP |
| Connection Monitoring | Ping Reboot, Scheduled Periodic Reboot |
| Firewall features | Port forward, trafﬁc rules, SNAT, Zone forwarding, Parental Control, Masquerading, MSS Clamping, Loopback IP, Loopback Rules. |
| Attack Prevention | SYN Flood Protection, Drop Invalid Packets |
| DHCP | Static and Dynamic |
| Multi-WAN Conﬁguration | Conﬁgurable Failover, load balancing between Cellular WAN, Wi-Fi WAN and Wired. |
| VLAN | Tagged VLAN, Untagged VLAN |
| VPN | Open VPN, IPsec, GRE, L2TP, PPTP, Wireguard, Zero Tier, IPIP Tunnel |
| Open VPN | TUN/TAP with TLS Certiﬁcates |
| IPsec | IKEv1, IKEv2, Aggressive/Main Mode |
| IPsec Encryption | 3DES, AES128, AES192, AES256 |
| IPsec Authentication | SHA1, MD5, SHA256, SHA384, SHA512 |
| IPsec DH Group | MODP768, MODP1024, MODP1536, MODP2048, MODP3072, MODP4096 |
| PPTP/L2TP | Server & Client with/without MPPE Encryption |
| Monitoring Application | Modem Monitoring with ICMP Echo, Router Monitoring with ICMP Echo |
| Time Synchronization | NTP Client, RTC, Manual |
| Diagnostic | Ping, Traceroute, Modem Debug Info, Iperf, Download Logs |
| Other Features | MAC binding, Connection Diagnostics, URL Filtering, Router Health Monitoring, Package Manager |

Remote Management

| NMS | Remote Monitoring & Management Platform for SILBO Networking products |
| --- | --- |
| NMS VPN | Remote Access of SILBO and non-SILBO Networking Devices |
| TR069 | TR069 Client for CPE Management |
| SMS Commands | Reboot Command, Uptime Command |

Operating System

| Operating System | Open-WRT based Linux OS |
| --- | --- |
| Supported Languages | Shell scripting, C, C++ |
| Development tools | Standard Packaging SDK package with built environment provided (Optional) |

ODU-MT7621-01 Outdoor Cellular Router with ABS/PC IP66 Housing , 1x External Antenna, 1 internal antenna, Mounting Kit.

*Star-marked features on the SILBO ODU-MT7621-Outdoor cellular router are accessible only after opening Top panel/ cover of the IP66 housing.

SPATIAL MEASUREMENT:

Top View

224 mm

140 mm mm

*All rights reserved, subject to change without prior notice.

Get in touch:

sales@invendis.com

www.silbonetworks.com

Invendis Technologies India Private Limited

No. 230, 1st Cross, 38th Main, BOOHBCS Layout, BTM 2nd Stage, Bangalore – 560 068